PAN-OS 10 Security Zones

I found the Security zone is very crucial feature, which segregates the different network segments in zones regulating the inbound outbound traffic and allowing them securely communicate/interact with each other through policies.

A security zone is a group of one or more physical or virtual firewall interfaces and the network segments connected to the zone’s interfaces.

Zones prevent uncontrolled traffic from flowing through the firewall interfaces into your network because firewall interfaces can’t process traffic until you assign them to zones. The firewall applies zone protection on ingress interfaces.

A firewall interface can belong to one zone only. You can connect more than one firewall interface to a zone to increase bandwidth, but each interface can connect to only one zone.

The more granular you make each zone, the greater the control you have over the traffic that accesses each zone, and the more difficult it is for malware to move laterally across the network between zones.

Dividing your network into zones helps to create a Zero Trust architecture that executes a security philosophy of trusting no users, devices, applications, or packets, and verifying everything.

Traffic can’t flow between zones unless a security policy rule allows it and the zones are of the same zone type (Tap, virtual wire, L2, L3, Tunnel, or External). For example, a security policy rule can allow traffic between two L3 zones, but not between an L3 zone and an L2 zone. The firewall logs traffic that flows between zones when a security policy rule permits interzone traffic.

The protection requirements for different functional areas and groups may also differ. For example, a zone that handles a large amount of traffic may require different flood protection thresholds than a zone that normally handles less traffic. The ability to define the appropriate protection for each zone is another reason to segment the network. What appropriate protection is depends on your network architecture, what you want to protect, and what traffic you want to permit and deny.

# Features Introduced in PAN-OS 11.0 - Cloud Identity Features

The new feature I found very interesting is Cloud Identity Engine. As my goal is to become cloud security engineer. The Cloud Identity Engine provides both user identification and user authentication for a centralized cloud-based solution in on-premise, cloud-based, or hybrid network environments - to quickly move towards a Zero Trust security posture. It allows you to write security policy based on users and groups, not IP addresses, and helps secure your assets by enforcing behavior-based security actions.

To provide user, group, and computer information for policy or event context, Palo Alto Networks cloud-based applications and services need access to your directory information. The Cloud Identity Engine, a secure cloud-based infrastructure, provides Palo Alto Networks apps and services with read-only access to your directory information for user visibility and policy enforcement.

It also provides the flexibility to adapt to changing security needs and users by making it simpler to configure an identity source or provider in a single unified source of user identity, allowing scalability as needs change.

The authentication component of the Cloud Identity Engine allows you to configure a profile for a SAML 2.0-compliant identity provider (IdP) that authenticates users by redirecting their access requests through the IdP before granting access. You can also configure a client certificate for user authentication.

The Cloud Identity Engine simplify identity across infrastructure and enable easy authentication and authorization through the cloud. Easily apply these controls everywhere in your enterprise: data center, campus, public cloud, branches, and remote users.